FORMAN CHRISTIAN COLLEGE

(A CHARTERED UNIVERSITY)

CSCS 306 A

FA24

Lab 9 Report

Syeda Manal Ammad - 251606966

Daim Bin Khalid – 251686775

Hafsah Shahbaz – 251684784

Introduction

This lab aimed to implement a real-time data collection and transmission system using an ESP32 microcontroller. The focus was on utilizing Wi-Fi capabilities to connect to an NTP server for time synchronization and a Google Web App endpoint for data logging. These tasks emphasized fundamental IoT concepts, including network communication, time synchronization, and serial debugging.

Task 1 concentrated on establishing a Wi-Fi connection and retrieving the current time from an NTP server. Task 2 extended this by integrating a Google Apps Script endpoint to send data for cloud-based logging.

Task 1: Wi-Fi and Time Synchronization

Functions

- Connected the ESP32 to a specified Wi-Fi network using the SSID and password.
- Configured the ESP32 to synchronize with an NTP server to retrieve the current time.
- Displayed the synchronized time on the Serial Monitor for real-time feedback.

Algorithms and Logic

Setup Phase:

1. Wi-Fi Initialization:
   - The ESP32 was configured to connect to the network using `WiFi.begin()`.
   - The connection status was checked in a loop until successful.

2. Time Synchronization:
   - The NTP server (`pool.ntp.org`) was used for accurate time retrieval.
   - Timezone and daylight saving offsets were configured using `configTime()`.

Loop Phase:

1. Retrieve local time using `getLocalTime()`.
2. Display the current time on the Serial Monitor.

Code Breakdown

#include "WiFi.h"
#include "time.h"

const char* ntpServer = "pool.ntp.org";
const long gmtOffset_sec = 18000; // GMT+5 for Pakistan
const int daylightOffset_sec = 3600;

const char* ssid = "Your_SSID";
const char* password = "Your_Password";

void setup() {
    Serial.begin(115200);
    WiFi.begin(ssid, password);
    while (WiFi.status() != WL_CONNECTED) {
        delay(500);
        Serial.print(".");
    }
    Serial.println("\nWi-Fi Connected");
    configTime(gmtOffset_sec, daylightOffset_sec, ntpServer);
}

void loop() {
    struct tm timeinfo;
    if (getLocalTime(&timeinfo)) {
        Serial.println(&timeinfo, "%A, %B %d %Y %H:%M:%S");
    } else {
        Serial.println("Failed to obtain time");
    }
    delay(1000);
}

Output

- Successful Wi-Fi connection confirmation.
- Real-time display of the synchronized time.

Task 2: Data Transmission to Google Apps Script

Functions

- Send data, including a counter and the current timestamp, to a Google Apps Script endpoint.
- Log the response from the server in the Serial Monitor for validation.

Algorithms and Logic

Setup Phase:

1. Google Web App ID:
   - The script URL was stored in a variable for HTTP requests.

Loop Phase:

1. Construct an HTTP POST request to send the counter and timestamp.
2. Print the server’s response for debugging.
3. Increment the counter and introduce a delay for periodic updates.

Code Breakdown

#include <HTTPClient.h>

String GOOGLE_SCRIPT_ID = "Your_Script_ID";
int count = 0;

void loop() {
    if (WiFi.status() == WL_CONNECTED) {
        HTTPClient http;
        String url = "https://script.google.com/macros/s/" + GOOGLE_SCRIPT_ID + "/exec?count=" + count;
        http.begin(url);
        int httpResponseCode = http.GET();
        if (httpResponseCode > 0) {
            String response = http.getString();
            Serial.println("Response: " + response);
        } else {
            Serial.println("Error in HTTP request");
        }
        http.end();
    } else {
        Serial.println("Wi-Fi Disconnected");
    }
    count++;
    delay(5000);
}

Output

Data transmission confirmed by server responses in the Serial Monitor.