FORMAN CHRISTIAN COLLEGE (A CHARTERED UNIVERSITY)

CSCS306 - A

Lab 5

Syeda Manal Ammad - 251606966

Hafsah Shahbaz – 251684784

Daim Bin Khalid – 251686775

Introduction

This lab project focuses on designing an automated lighting control system using an Arduino microcontroller, PIR motion sensors, and Light Dependent Resistors (LDRs). The goal is to develop a system that activates lights only when needed, improving energy efficiency by responding to both motion and ambient light conditions.

Task 1 involves setting up a basic motion-activated lighting system, where an LED is controlled by a PIR sensor that detects movement, simulating a simple automated lighting scenario. An LDR monitors ambient light levels, laying the groundwork for more complex lighting controls.

Task 2 expands on this by creating a lighting system for staircases that activates in low-light conditions when motion is detected, remaining on for a set duration. This task demonstrates a practical application of automated lighting, where lights are triggered based on environmental factors, optimizing energy use.

Task 1: LDR and PIR Motion Sensor Control for LED

Functions

- analogRead(): Reads the value from an analog sensor (LDR in this case) and returns a number between 0 and 1023, which represents the light intensity.

- digitalRead(): Reads the state of a digital pin (used for detecting motion from the PIR sensor) and returns either HIGH or LOW.

- digitalWrite(): Sets a digital pin (connected to the LED) to either HIGH (turn on) or LOW (turn off).

- delay(): Pauses the program for a specified number of milliseconds. In this task, it's used to control the duration for which the LED stays on or off.

- Serial.begin(): Initializes serial communication at a specified baud rate (9600 in this case), enabling output to the Serial Monitor.

- Serial.println(): Sends the data (LDR value in this case) to the Serial Monitor for debugging or observation.

Libraries

- Arduino.h: This is the default library included in every Arduino sketch. It provides essential functions like pinMode(), digitalWrite(), digitalRead(), analogRead(), and delay().

Code

// Define the pins for each component

int ldrPin = A0;       // LDR connected to analog pin A0

int pirPin = 2;        // PIR sensor connected to digital pin 2

int ledPin = 13;       // LED connected to digital pin 13

int ldrValue = 0;      // Variable to store LDR reading

void setup() {

pinMode(ldrPin, INPUT);      // Set the LDR pin as an input

pinMode(pirPin, INPUT);      // Set the PIR sensor pin as an input

pinMode(ledPin, OUTPUT);     // Set the LED pin as an output

Serial.begin(9600);          // Start serial communication at 9600 bps

}

void loop() {

ldrValue = analogRead(ldrPin);     // Read the LDR value (0-1023)

Serial.println(ldrValue);          // Print the LDR value to the serial monitor

if (digitalRead(pirPin) == HIGH) { // If motion is detected

digitalWrite(ledPin, HIGH);    // Turn LED on

delay(5000);                   // Keep LED on for 5 seconds

digitalWrite(ledPin, LOW);     // Turn LED off

}

delay(1000);                       // Wait 1 second before next loop

}

Explanation

- Pin Configuration:

  - ldrPin is connected to analog pin A0, where the LDR measures ambient light levels.

  - pirPin is connected to digital pin 2, where the PIR sensor detects motion.

  - ledPin is connected to digital pin 13, controlling an LED that acts as a visual alert.

- Serial Communication:

  - Serial.begin(9600); initializes serial communication, allowing us to monitor LDR values in the Arduino IDE's serial monitor.

- Main Code Logic:

  - Reading LDR Value: The LDR measures ambient light intensity, with values from 0 (dark) to 1023 (bright).

  - Motion Detection: The PIR sensor outputs HIGH when motion is detected, triggering the LED.

  - LED Control: When motion is detected, the LED turns on for 5 seconds, then turns off.

  - Loop Delay: Adds a 1-second delay to stabilize the loop, reducing unnecessary checks and ensuring smooth operation.

Results

In this setup, the LED lights up whenever the PIR sensor detects motion, regardless of the ambient light level measured by the LDR. The LDR value is printed to the serial monitor, allowing us to observe changes in light intensity.

Task 2: Automated Staircase Lighting System

Functions

- analogRead(): Reads the LDR value (ambient light level) to determine if the environment is dark enough to turn on the light.

- digitalRead(): Detects motion from the PIR sensor, indicating if the user is present in the area.

- digitalWrite(): Turns the LED on or off depending on the conditions set by the PIR sensor and the LDR reading.

- millis(): Returns the number of milliseconds since the Arduino board began running the current program. This is used to track the duration that the LED should stay on.

- Serial.begin(): Initializes serial communication to enable debugging and observation of the LDR readings on the Serial Monitor.

- Serial.println(): Prints the LDR value to the Serial Monitor for real-time feedback.

Libraries

- Arduino.h: The core Arduino library that includes all the essential functions required to control the hardware like LEDs, sensors, and serial communication.

Code

// Define the pins and variables

int ldrPin = A0;               // LDR connected to analog pin A0

int pirPin = 2;                // PIR sensor connected to digital pin 2

int ledPin = 13;               // LED connected to digital pin 13

int threshold = 500;           // Threshold value for the LDR

long duration = 120000;        // Duration (120 seconds) for the LED to stay on

long startTime;                // Variable to store the time when the LED is turned on

void setup() {

pinMode(ldrPin, INPUT);    // Set the LDR pin as an input

pinMode(pirPin, INPUT);    // Set the PIR sensor pin as an input

pinMode(ledPin, OUTPUT);   // Set the LED pin as an output

Serial.begin(9600);        // Start serial communication at 9600 bps

}

void loop() {

int ldrValue = analogRead(ldrPin);  // Read the LDR value (0-1023)

int pirState = digitalRead(pirPin); // Read the PIR state (HIGH or LOW)

if (pirState == HIGH && ldrValue < threshold) { // Motion detected and low light

digitalWrite(ledPin, HIGH);       // Turn LED on

startTime = millis();             // Store the current time as start time

}

if ((millis() - startTime) > duration) { // If 120 seconds have passed

digitalWrite(ledPin, LOW);        // Turn LED off

}

Serial.print("LDR Value: ");

Serial.println(ldrValue);             // Print LDR value to serial monitor

delay(1000);                          // Wait 1 second before next loop

}

Explanation

- Pin Configuration:

  - ldrPin, pirPin, and ledPin are configured similarly to Task 1.

  - threshold is set to 500, so the LED only turns on when light levels are below this value (indicating darkness).

  - duration is set to 120,000 milliseconds (120 seconds), representing the time the LED stays on after motion is detected.

- Main Code Logic:

  - LDR and PIR Reading: ldrValue captures light intensity, and pirState detects motion.

  - Light Activation: If the PIR detects motion and light levels are below the threshold, the LED turns on and startTime is set to the current time.

  - Auto Turn-Off: The millis() function checks if 120 seconds have passed since startTime. If true, the LED turns off.

  - Serial Monitoring: The LDR value is printed for observation.

Results

This setup creates a lighting system that only activates in low-light conditions when motion is detected. After turning on, the LED remains active for 120 seconds before turning off, even if motion continues, making it energy-efficient and suitable for scenarios like staircase lighting.