FORMAN CHRISTIAN COLLEGE

(A CHARTERED UNIVERSITY)

CSCS 306 - A

FALL 24

Lab 10 Report

Daim Bin Khalid - 251686775

Syeda Manal Ammad - 251606966

Zaki Babar - 251687387

12th December 2024

Introduction

This lab focused on creating a system to log data from an ESP32 microcontroller to a Google Sheet using Google Apps Script. The objective was to toggle an LED’s state using a push-button and to record the state changes (ON/OFF) along with corresponding date and time in a cloud-based Google Sheet. The project emphasized the integration of embedded systems with cloud-based services, real-time data logging, and practical usage of ESP32 capabilities.

Functions

ESP32 Implementation

- Button State Detection:

  - Read the button state to toggle the LED state.

  - Implement debounce logic to handle false button readings.

- Google Sheet Integration:

  - Utilize WiFi connectivity of the ESP32 to send HTTP GET requests to a Google Apps Script Web App.

  - Include parameters: ledstatus (1 or 0), date, and time.

Google Apps Script

- Append received parameters (ledstatus, date, time) to a Google Sheet.

- Deploy the script as a Web App with public access.

Algorithm and Logic

ESP32 Code

Setup Phase:

- Initialize serial communication for debugging.

- Configure GPIO pins for LED and button.

- Connect to WiFi and synchronize time using an NTP server.

Loop Phase:

- Detect button presses and toggle the LED state.

- On state change, send an HTTP GET request with LED state, date, and time.

- Log the response for debugging.

Google Apps Script

- Parse incoming parameters: ledstatus, date, and time.

- Append the data as a new row in the Google Sheet.

- Return a success message.

Code Implementation

Arduino Code

#include "WiFi.h"

#include <HTTPClient.h>

#include "time.h"

const char* ntpServer = "pool.ntp.org";

const long gmtOffset_sec = 18000; // Adjust for your timezone

const int daylightOffset_sec = 3600;

const char* ssid = "Lounge";

const char* password = "redbrown7";

String GOOGLE_SCRIPT_ID = "AKfycbxR7p9LIBckfXaVeGAYhuNuU02_c4-Aao7_dL8kxvJsL-9bDxd5HwvJrmnKqpQUBVTo";

#define LED_PIN 2

#define BUTTON_PIN 15

bool ledState = false;  // Current state of the LED

bool lastLedState = false; // To track state changes

void setup() {

Serial.begin(115200);

delay(1000);

Serial.println("\nStarting setup...");

// Configure GPIO pins

pinMode(LED_PIN, OUTPUT);

pinMode(BUTTON_PIN, INPUT_PULLUP);

// Connect to WiFi

Serial.print("Connecting to WiFi: ");

Serial.println(ssid);

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\nWiFi connected.");

configTime(gmtOffset_sec, daylightOffset_sec, ntpServer);

Serial.println("Time synchronization initialized.");

}

void uploadToGoogleSheet(bool state) {

if (WiFi.status() == WL_CONNECTED) {

struct tm timeinfo;

if (!getLocalTime(&timeinfo)) {

Serial.println("Failed to obtain time.");

return;

}

// Format date and time separately

char dateBuff[15], timeBuff[10];

strftime(dateBuff, sizeof(dateBuff), "%Y-%m-%d", &timeinfo);

strftime(timeBuff, sizeof(timeBuff), "%H:%M:%S", &timeinfo);

// Create the URL

String url = "https://script.google.com/macros/s/" + GOOGLE_SCRIPT_ID + "/exec?ledstatus=" + String(state ? "1" : "0") +

"&date=" + String(dateBuff) +

"&time=" + String(timeBuff);

// Send HTTP request

HTTPClient http;

http.begin(url);

}

}

void loop() {

bool buttonState = !digitalRead(BUTTON_PIN); // Inverted logic for INPUT_PULLUP

// Toggle LED based on button state

if (buttonState) {

if (!ledState) { // Change state only if it’s off

ledState = true;

digitalWrite(LED_PIN, HIGH);

Serial.println("LED turned ON.");

}

} else {

if (ledState) { // Change state only if it’s on

ledState = false;

digitalWrite(LED_PIN, LOW);

Serial.println("LED turned OFF.");

}

}

// Upload only on state change

if (ledState != lastLedState) {

uploadToGoogleSheet(ledState);

lastLedState = ledState;

}

delay(500);

}

Google Apps Script

var sheet_id = "1g0_lcIP1TWKY2727e1yCCflBdPvNy-UK7Yg7DGv60NA";

var sheet_name = "lab10";

function doGet(e) {

var ledStatus = e.parameter.ledstatus; // LED status: 1 (ON) or 0 (OFF)

var date = e.parameter.date;       // Date

var time = e.parameter.time;       // Time

// Open the sheet

var ss = SpreadsheetApp.openById(sheet_id);

var sheet = ss.getSheetByName(sheet_name);

// Append the data

sheet.appendRow([ledStatus, date, time]);

// Return success response

return ContentService.createTextOutput("Success");

}

Output

The system successfully logged LED state changes to the Google Sheet, as shown below: